SOP for using the oxyhydrogen torch

Before using the torch prospective users must be trained by Chris Thomas (Ø-138) (for NAFUMA personnel) or Anja O. Sjåstad (Ø-130) (for other groups)

- Wear face shield (welding face shield or welding glasses and ordinary face shield)

- Check that fume hood is on

- Check that all valves for both O2 and H2 are closed (gas cylinders, regulators and torch).

Open for H2-gas flow

- Open main valve on H2-gas cylinder.

- Check the pressure of first gauge on the H2-regulator (minimum allowed cylinder pressure 30 kPa/cm2).

- Open slightly main reduction valve on the regulator (turn clockwise). Check pressure of the second gauge on the regulator (max allowed pressure 1.5 kPa/cm2).

Ignite flame on torch

- Open the hydrogen valve on the torch, and ignite fire by match or lighter.

- Adjust hydrogen flow to obtain an orange flame.

- Open carefully the oxygen valve on the torch and adjust flow to blue colour for use.

- Open for O2-gas flow

- Open main valve on O2-gas cylinder.

- Open slightly the valve (counter clockwise), and check the increase in pressure of first and second gauges on the regulator. Max operating pressure for second gauge is 5 bar.

Adjust flame on torch for a short break (max 5 minutes)

- Close oxygen valve on the torch

- Adjust hydrogen flow on the torch to give an orange flame

- Reset H2/O2 flame (after break) by carefully open oxygen valve on the torch and adjust flow to give blue colour.

Close H2/O2 gas flow at end of experiment

- Close main valves on O2- and H2-gas cylinders. The remaining gases in the lines will be treated in steps 2-5 below.

- Burn remaining H2-gas in the line until the pressure of both first and second gauges on the H2 cylinder decreases to zero. Close H2 valve on the torch.

- Release remaining O2-gas until pressure of both first and second gauges on the O2 cylinder becomes zero.

- Close all valves on both H2 and O2 lines and assure both gas cylinders are in closed position.Do not over-tighten needle values.

Remember to tidy workspace after use

Report any problems or concerns to Christopher I. Thomas (Ø-138) or Anja O. Sjåstad (Ø-130)

SAFE JOB ANALYSIS (SJA)

Name of unit: Department of Chemistry

This SJA concerns: hydrogen torch in room Ø150

Date: 1/11-2010, Agnieszka Gorzkowska-Sobas, Hiroshi Okamoto, Helmer Fjellvåg

Date: 20/3-2012: Christopher Ian Thomas, Anja O. Sjåstad

| Subtask | Possible cause of undesired event | Precautions to be taken |
| --- | --- | --- |
| Hydrogen gas | Flammability/Ignitability | Turn on fume hood |
|  |  | Hydrogen gas must be always burned out. |
| Sealing quartz | Damage to the eyes | Do not look at melting part of quartz by naked eye |
|  | Damage to the face | Use face-shield for protection |
|  | Skin burns | Keep tidy work place, and maintain good housekeeping |
|  | Explosion | Think about possible over pressure during sealing treatment Assure H2 gas cylinder have at least 30kPa/cm2 when starting up experiment |